Student name:………………………………………

# Question 3

Use the corresponding numbers and letters in the table below to answer the question.

For each environmental condition listed, select a conductor and an insulator that would work best for that condition. Then give a reason for your selection.

Please note, you can use the same conductor, insulator and reason more than once. You only need to choose one letter or number for each answer.

| Condition | Conductor (insert number) | Reason why conductor selected (insert letter) | Insulator (insert number) | Reason why insulator selected (insert letter) |
| --- | --- | --- | --- | --- |
| Heat |  |  |  |  |
| Moisture |  |  |  |  |
| Corrosive materials |  |  |  |  |
| Dust |  |  |  |  |
| Tension |  |  |  |  |
| Compression |  |  |  |  |
| Vibration |  |  |  |  |

|  | Conductor/Insulator |  | Conductor/Insulator |  | Conductor/Insulator |
| --- | --- | --- | --- | --- | --- |
| 1 | Copper | 7 | Brass | 13 | Glass |
| 2 | Silver | 8 | Gold | 14 | Mica |
| 3 | Aluminium | 9 | Lead | 15 | Oil |
| 4 | Tungsten | 10 | Tin | 16 | Ceramic porcelain |
| 5 | Carbon | 11 | Hard drawn copper | 17 | Hard rubber |
| 6 | Nichrome | 12 | Stranded copper | 18 | PVC |
|  | Reason for selection |  | Reason for selection |  | Reason for selection |
| A | High melting point | E | High tensile strength | I | Soft |
| B | Resistant to corrosion | F | Flexible | J | Crimping lugs and sleeves can be used |
| C | Resistant to oxidation | G | Withstand vibration | K | Can cope with compression |
| D | Unaffected by dust | H | Water resistant |  |  |

(ER 1.3)

© The Skills Organisation Incorporated 2018 	NZ Certficate in Electrical Engineering Theory and Practice (Trade) (Level 4)